Title: Reading Data from DHT11 Sensor and Writing to SD Card using ESP32

Components Used:

- Hardware Used:

  - ESP32

  - DHT11 Sensor

  - SD Card

- Software Used:

  - Arduino IDE

  - ESP32 Libraries (DHT, SD)

  - HTML for the web interface

Introduction:
This experiment demonstrates reading temperature and humidity data from a DHT11 sensor using an ESP32 and writing the data to an SD card. The ESP32 hosts a web interface to initiate the operations.

Block Diagram:

SD Card

ESP32

DHT11 Sensor

Flow Chart:

Pin Connections Table:

| ESP32 Pin | DHT11 Pin |
| --- | --- |
| GPIO 4 | Data |
| 3.3V | VCC |
| GND | GND |

| SD Card Pin | ESP32 Pin |
| --- | --- |
| CS | GPIO 5 |
| SCK | GPIO 18 |
| MOSI | GPIO 23 |
| MISO | GPIO 19 |
| VCC | 3.3V |
| GND | GND |

Procedure:

- Connect the DHT11 sensor to the ESP32.

- Connect the SD card module to the ESP32 as per the pin diagram.

- Install the required libraries in the Arduino IDE:

  - DHT

  - SD

- Create an Arduino sketch to read data from the DHT11 sensor and write to the SD card.

- Create an HTML page to interact with the ESP32.

Code:

Arduino (AmpServer.ino)

#include <SD.h>

#include <SPI.h>

#include <WiFi.h>

#include <DHT.h>

#define DHTPIN 4

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

File myFile;

const int chipSelect = 5;

const char* ssid = "OnePlus";

const char* password = "719622@Aa";

WiFiServer server(80);

void setup() {

Serial.begin(115200);

dht.begin();

if (!SD.begin(chipSelect)) {

Serial.println("SD card initialization failed!");

return;

}

Serial.println("SD card initialized.");

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(1000);

Serial.println("Connecting to WiFi...");

}

Serial.println("Connected to WiFi.");

server.begin();

}

void loop() {

WiFiClient client = server.available();

if (client) {

String request = client.readStringUntil('\r');

client.flush();

if (request.indexOf("/read_write_data") != -1) {

float h = dht.readHumidity();

float t = dht.readTemperature();

myFile = SD.open("data.txt", FILE_WRITE);

if (myFile) {

myFile.print("Humidity: ");

myFile.print(h);

myFile.print("%, Temperature: ");

myFile.print(t);

myFile.println(" *C");

myFile.close();

Serial.println("Data written to file.");

} else {

Serial.println("Failed to open file.");

}

client.print("HTTP/1.1 200 OK\r\nContent-Type: text/html\r\n\r\n");

client.print("<html><body><h1>Data written to SD card</h1>");

client.print("<p>Humidity: ");

client.print(h);

client.print("%</p><p>Temperature: ");

client.print(t);

client.print(" *C</p>");

client.print("<button onclick=\"readWriteData()\">Read & Write Data</button>");

client.print("<script>");

client.print("function readWriteData() {");

client.print("fetch('/read_write_data').then(response => response.text()).then(data => {");

client.print("document.getElementById('output').innerHTML = data;");

client.print("}).catch(error => {");

client.print("document.getElementById('output').innerText = 'Error: ' + error;");

client.print("});");

client.print("}");

client.print("</script>");

client.print("</body></html>");

}

delay(1);

client.stop();

}

}

How to Operate the Code:

- Setup:

  - Open the Arduino IDE.

  - Install the required libraries: DHT, SD, WiFi.

  - Connect the ESP32, DHT11 sensor, and SD card module as per the pin diagram.

- Upload the Code:

  - Copy the provided Arduino sketch into the Arduino IDE.

  - Update the WiFi credentials in the code (ssid and password).

  - Upload the code to the ESP32.

- Web Interface:

  - After uploading, open a serial monitor to get the IP address of the ESP32.

  - Open a web browser and enter the IP address.

  - Click the "Read & Write Data" button to read data from the DHT11 sensor and write it to the SD card. The result will be displayed on the web page.

Output: 
The temperature and humidity data from the DHT11 sensor are written to a text file on the SD card, and a confirmation is displayed on the web interface.

Conclusion: 
The experiment demonstrates reading data from a DHT11 sensor and writing it to an SD card using an ESP32.

Applications:

- Environmental monitoring systems.

- Data logging for weather stations.