Title: Writing Data to SD Card using ESP32 and Viewing Output via Serial Monitor

Components Used:

Hardware Used:

- ESP32

- SD Card Module

- SD Card

Software Used:

- Arduino IDE

- ESP32 Libraries (SD, SPI)

Introduction: This experiment involves writing data to a text file on an SD card using an ESP32 and verifying the data by reading it back and displaying it on the serial monitor.

Block Diagram:

Libraries Used

- SD.h:

  - Provides the functionality to read from and write to SD cards using the SD card slot on the ESP32 or an SD card module.

- SPI.h:

  - Provides the functionality to communicate with devices using the SPI protocol. Used here to interface with the SD card module.

Protocols Used

- SPI (Serial Peripheral Interface):

  - A high-speed serial communication protocol used for interfacing devices like SD cards, sensors, and displays. In this code, the SPI protocol is used to communicate with the SD card module. The SPI.h library provides the necessary functions for this communication.

Brief Overview of the Protocol

- SPI (Serial Peripheral Interface):

  - SPI is a four-wire communication protocol consisting of MOSI (Master Out Slave In), MISO (Master In Slave Out), SCK (Serial Clock), and SS (Slave Select).

  - It allows full-duplex communication between a master and one or more slave devices.

  - The master device controls the clock signal, and data is exchanged simultaneously between the master and the slave device.

Circuit Diagram:

Insert Circuit Diagram Here

Pin Connections Table:

| ESP32 Pin | SD Card Pin |
| --- | --- |
| GPIO 5 | CS |
| GPIO 18 | CLK |
| GPIO 23 | MOSI |
| GPIO 19 | MISO |
| 3.3V | VCC |
| GND | GND |

Procedure:

- Connect the SD card module to the ESP32 as per the pin diagram.

- Install the required libraries in the Arduino IDE:

  - SD

  - SPI

- Create an Arduino sketch to write data to the SD card and verify it by reading the data back.

Code:

#include <SD.h>

#include <SPI.h>

#define CS_PIN 5

void setup() {

Serial.begin(115200);

if (!SD.begin(CS_PIN)) {

Serial.println("SD card initialization failed!");

return;

}

Serial.println("SD card initialized.");

// Open file for writing

File myFile = SD.open("/data.txt", FILE_WRITE);

if (myFile) {

myFile.println("Hello, ESP32!");

myFile.close();

Serial.println("Data written to file.");

} else {

Serial.println("Failed to open file for writing.");

}

// Reopen file for reading

myFile = SD.open("/data.txt");

if (myFile) {

Serial.println("Reading from file:");

while (myFile.available()) {

Serial.write(myFile.read());

}

myFile.close();

} else {

Serial.println("Failed to open file for reading.");

}

}

void loop() {

// Nothing to do here

}

Explanation of the Code:

The provided code initializes an SD card, writes a string to a text file on the SD card, and reads the content back to display on the serial monitor.

- Libraries and Constants:

  - Includes necessary libraries for handling the SD card.

  - Defines a constant for the SD card chip select pin.

- Setup Function:

  - Initializes serial communication.

  - Initializes the SD card.

  - Opens a file on the SD card for writing and writes a string to it.

  - Closes the file after writing.

  - Reopens the file for reading and displays the content on the serial monitor.

- Loop Function:

  - Empty loop function as all tasks are performed in the setup.

How to Operate the Code:

- Setup:

  - Open the Arduino IDE.

  - Install the required libraries: SD, SPI.

  - Connect the ESP32 and SD card module as per the pin diagram.

- Upload the Code:

  - Copy the provided Arduino sketch into the Arduino IDE.

  - Upload the code to the ESP32.

- Prepare SD Card:

  - Insert the SD card into the SD card module connected to the ESP32.

- View on Serial Monitor:

  - After uploading, open the Serial Monitor in the Arduino IDE to view the data written to and read from the SD card.

Output:

Conclusion: The experiment demonstrates writing data to an SD card using an ESP32 and verifying the data by reading it back and displaying it on the serial monitor.

Applications:

- Data logging and storage for IoT applications.

- Storing configuration or calibration data for embedded systems.