Date: 30/05/2024

Day: Thursday

Experiment no: 15

- Title – Measurement of Temperature using DHT11 sensor using IoT development kit.

- Introduction-

In this project, we explore data transmission using Free Space Optics (FSO) with an ESP32 microcontroller. FSO technology allows for the wireless transfer of data through light, utilizing a laser and a light-dependent resistor (LDR) for sending and receiving signals. By employing the DOIT ESP32 DevKit V1, a powerful microcontroller board based on the ESP32 System-on-Chip (SoC), we can implement a reliable FSO communication system. This report details the setup, implementation, and results of using the ESP32 for transmitting binary data via a laser and receiving it with an LDR, demonstrating the capabilities and practical applications of FSO technology.

- Software used:

  - Arduino IDE

- Hardware used:

  - Transmitter side:

    - ESP 32

    - Photo Diode

  - Receiver side:

    - ESP 32

    - LDR

    - 1kΩ Resistor

| Component | Specification |
| --- | --- |
| Transistor | N-channel MOSFET (likely 2N7000) |
| Resistor | 330 kΩ resistor |
| Laser | Laser diode (likely 1C2A) |

Table 1.1 Components and its specifications of Transmitter side

| Component | Specification |
| --- | --- |
| LDR Symbol | Light-dependent resistor (LDR) |
| Resistor | 1kΩ |

Table 1.2 Components and its specifications of Receiver side

| ESP 32 WROOM Feature | Specification |
| --- | --- |
| Microprocessor | Tensilica Xtensa LX6 |
| Cores | Dual Core |
| Clock Frequency | 160 MHz (adjustable to 240 MHz) |
| Memory | * SRAM: 512 KB * Flash: Varies depending on module |
| Wireless Connectivity | Wi-Fi 802.11 b/g/n |
| Bluetooth | Yes (4.2 and Low Energy) |
| Operating System | Bare-metal, RTOS (Real-Time Operating System) |
| Security Features | WPA/WPA2 encryption, Secure Boot |
| Power Supply | 3.3V |
| Current Consumption | Higher (down to 20uA deep sleep) |
| Operating Temperature | -40°C to +125°C |
| Dimensions | Varies depending on module (typically larger than ESP8266 module) |
| Interfaces | SPI, I2C, UART, ADC, DAC, PWM, High-speed SPI, Touch Sensor (may vary) |

Table 1.3 Specifications of ESP 32 WROOM Module.

- Circuit Diagram:

Figure 1.2 Receiver of FSO

Figure 1.1 Transmitter of FSO

| Component | ESP32 Pin | Description |
| --- | --- | --- |
| Transmitter |  |  |
| Laser Diode | Any GPIO pin (e.g., 25) | Connects the positive (+) lead of the laser diode to the ESP32 pin. A current-limiting resistor (typically 100 ohms to 470 ohms) is recommended in series with the laser to prevent damage. |
| Ground | GND | Connects the negative (-) lead of the laser diode and the current-limiting resistor (if used) to the ESP32 ground pin. |
| Receiver |  |  |
| Phototransistor (LDR) | Any analog input pin (e.g., 34) | Connects the signal pin of the LDR to the ESP32 analog input pin. |
| Resistor (optional) | Any GPIO pin (e.g., 26) | A pull-up resistor (typically 10k ohms to 47k ohms) can be connected between the LDR signal pin and the ESP32 power pin (3.3V) to improve signal stability (optional). |
| Ground | GND | Connects the ground pin of the LDR to the ESP32 ground pin. |

Table 2. Pin connections at transmitter and receiver side

- Block Diagram:

ESP 32

+

LDR

ESP 32

+

Photodiode

Figure 2. Block Diagram of the FSO Data Transmission.

- Flowchart

  - Transmitter:

  - Receiver:

- Source Code:

- Transmitter code:

#include <Arduino.h>

const int laserPin = 17; // GPIO pin for Laser Diode

int numberToSend = 54; // The number you want to send

void setup() {

Serial.begin(115200);

pinMode(laserPin, OUTPUT);

Serial.println("FSO Transmitter Initialized");

}

void sendBit(bool bit) {

digitalWrite(laserPin, bit ? HIGH : LOW);

delay(100); // Delay to separate bits

}

void loop() {

// Send start bits

for (int i = 0; i < 8; i++) {

sendBit(true);

}

// Transmit numberToSend value

for (int i = 0; i < 8; i++) {

sendBit(numberToSend & (1 << i));

}

// Send stop bits

for (int i = 0; i < 8; i++) {

sendBit(false);

}

Serial.print("Data Sent: ");

Serial.println(numberToSend);

delay(1000); // Delay for 1 second before sending again

}

- Receiver code:

#include <Arduino.h>

const int phototransistorPin = 34; // GPIO pin for Phototransistor

const int ledPin = 4; // GPIO pin for LED

const int threshold = 900; // Threshold for analog reading

void setup() {

Serial.begin(115200);

pinMode(ledPin, OUTPUT);

Serial.println("FSO Receiver Initialized");

}

bool readBit() {

int value = analogRead(phototransistorPin);

bool bitValue = value > threshold;

delay(100); // Delay to synchronize with transmitter

return bitValue;

}

void loop() {

int receivedData = 0;

// Wait for start bits

for (int i = 0; i < 8; i++) {

if (!readBit()) {

Serial.println("Waiting for Next Data->");

return;

}

}

Serial.println(); // New line after start bits

// Receive 8 data bits

for (int i = 0; i < 8; i++) {

if (readBit()) {

receivedData |= (1 << i);

}

}

Serial.println(); // New line after data bits

// Check stop bits

bool stopBitsCorrect = true;

for (int i = 0; i < 8; i++) {

if (readBit()) {

stopBitsCorrect = false;

break;

}

}

if (stopBitsCorrect) {

Serial.print("Binary Data Received: ");

Serial.println(receivedData, BIN);

Serial.print("Decimal Data Received: ");

Serial.println(receivedData);

// Display received data on LED

for (int i = 0; i < 8; i++) {

digitalWrite(ledPin, (receivedData & (1 << i)) ? HIGH : LOW);

delay(500); // Display each bit for 500 ms

}

} else {

Serial.print(".");

}

delay(1000); // Delay for 1 second before the next reception

}

- Output:

Figure 3.1 Transmitter output

Figure 3.2 Receiver output

- Applications:

- Short-Range Data Transfer

- Infrared Remote Controls

- Learning Optoelectronics

- Digital Communication Protocols

- Wireless Sensor Networks

- Internet of Things (IoT)

- Secure Communication

- Remote Monitoring and Control

- Prototype Development

- Performance Testing

- Temporary Networks

- Peripheral Communication